TIMER SUPPORT

Time management is driven by a Z80-CTC which is programmed to

interrupt every millisecond.  There is one software controlled

time field.  TIMER is a simple two-byte hex counter that is

incremented every millisecond.  It is allowed to overflow to zero

whenever it feels the need to do so.  More sophisticated timer

support is provided by the time-interval queue.

Timer queue services provide two basic modes of support, set-

timer-exit, and set-timed-wait.  A task (or a routine running in

task mode) may request that the current task be placed in a wait

for a specified period of time.  Any routine may use the set-

timer-exit function.  In this case, the caller specifies the

address of a routine that will receive control when the period of

time expires.  This exit routine gets control disabled for

interrupts, with IX pointing to the expired timer queue element

(TQE).  The caller may use the TQE to pass data to the exit

routine.  It is the responsibility of the exit routine to dispose

of the TQE, either by freeing it, or by putting it on another

queue.  Since the exit routine runs disabled, it should be brief.

Typically it will set a flag or two, post a task, free the TQE,

and return to the timer interrupt handler.

The time interval may be specified in any one of three ranges.  A

short-range interval may be 1 to 65,535 milliseconds.  A long-

range interval may be from 1 to 65,535 minutes.  For an

intermediate-range interval, the caller specifies a time interval

in milliseconds (from 1 to 65,535) and a repetition count (from 2

to 255).  When the time interval has been repeated the required

number of times, the timer expires.  For example, a time interval

of 10.5 minutes requires 21 repetitions of 30,000 milliseconds.

All timer intervals are accurate to the nearest millisecond.

There is only one timer queue.  During each timer interrupt, the

first TQE in the time-interval queue is decremented.  Any TQE

that decrements to zero is removed from the queue, and the exit

routine for that TQE is given control.  After the exit returns

control, the next TQE is also checked, since it might specify a

count of zero.  This is due to the way the TQE's are placed on

the queue.  The time-interval queue is maintained in sequence.  A

new entry is compared with each existing TQE.  Its value is

decremented by the count in each existing TQE until it is no

longer larger than the next TQE.  It is chained into the queue in

front of this larger TQE and the new TQE's remaining count is

subtracted from the count of the next TQE.  It is therefore

possible for a TQE to have a count of zero.  This TQE will expire

at the same time as the one in front of it.

For intermediate and long range timers, the caller's exit address

is saved in the TQE, and a special timer exit is given control

when each intermediate timer expires.  This routine decrements

the repetition count in the TQE and sets a new timer for the next

intermediate interval.  When the repetition count reaches one,

the caller's original exit address is restored, so that when the

final timer expires, this exit will receive control.

Although the primary construct for a work-unit is the task, it is

possible to implement a different method of performing a

function.  Through the use of the set-timer-exit facility, it is

possible to set up either a finite-step or an un-ending state

machine.  The BEEP function is an example of the finite-step

variety.  When the BEEP routine is called, it turns on the alarm

and sets a timer exit.  When the exit gets control, it decrements

the beep count which is saved in the TQE.  If the count is not

zero, it sets the alarm to its opposite state (on/off) and re-

queues its TQE for another interval.  When the count decrements

to zero, the TQE is freed and the process terminates.  In other

applications, the exit could change the exit address and/or the

timer value for the next timer exit.  If the TQE were always re-

queued, the process would never terminate.

#####################################################################################################