this file is best viewed with WORDPAD.EXE and a screen res of 800 x 600 .

Dear Manufacturer,

Here is a list of all the components involved in the design. I require you to design a suitable Surface Mount PCB for the main board,

and provide suitable graphics and packageing. the packageing graphics I will leave to your discretion, make it interesting and colourfull

please. The words 'PC DESCATOC Card' should be used extensively on the packageing.

should you require a designed owners manual let me know, although, this will not be needed until I give you the go ahead to

manufacture. Unlike large companies, you will recieve full payment from me before you buy in components and tool up, as a matter of

a goodwill gesture. I hope you find this pleaseing.

if their is anything at all amiss, don't hesitate to contact me about any problems. you can contact me via various means as listed below.

provided in this ZIP file are various picture files to assist you in makeing a rapid assesment of the sort of SMT components that could

fit on the new PCB of your design. Of course, you have ultimate power over what you want to mount on the SMT PCB, so

component dimensions are really entirely at your discretion.

This card is fully patented. the number is:	GB 2 339 931 A

Mechanical information on the ISA Bus can be found in the file ISAinfo.JPG.

The full circuit diagram is in the file Schematc.JPG.

Full software for the PC, and for you to put into the EPROM, will be provided when or if production starts.

Finally, when I give you the go ahead, it would give me the greatest pleasure if I could see you make my Card in the production area ?

I look forward to that.  :-)

Thank you,

Jim Delaney, Sheffield, UK

Telephone:             0114 2499802

24 hrs BBS:           0114 2257296 Dial for 60 secs then hang up for 60 secs, redial. Use W95 Hyperterminal or similar DOS Terminal software.

24 hrs FAX:           0114 2257305 Dial for 60 secs then hang up for 45 secs, redial and send with your FAX device.

Linktel 24hrs

Video Phone:         0114 2214340 Dial for 60 secs then hang up for 60 secs, redial with your Linktel VPhone.

Get Linktel from:

http://www.cyberlink.com.tw

ALL the above are on a SINGLE phone line connected to a single Modem and PC, and are all operated on by my DESCATOC Card prototype. ;-)

E-Mail:                    james@gerrard61.freeserve.co.uk

If you let me license it to you, so you can produce it and market it, so much the better for me, luv for you all would be in my eyes a long time !! :)

If you would re consider a licenseing deal, and you are uncertain of the Market, go to someone like Creative Labs or Aztech or IBM of whom,

I am yet to contact, and get from them the status of the PC accessories market. If you like it, great!, if you don't, at least you have investigated

it with information from the main market players.

FULL COMPONENT DESCRIPTION

MAIN DESCATOC CARD

SEMICONDUCTORS						Purchase  Information

IC1,                      1  x	Z80B Central Processor 4 or 6Mhz			available as a QFP from zilog http://www.Zilog.com as

Z84C0010FEC. will send pin out info when I Rx from Zilog.

should be available as a PLCC package also from Zilog.

IC3,                      1  x	DS1220AB150 2k NVSRAM Memory + back up power	RS part No. 248-2943. pin out info in DS1220AB.JPG

IC2,                      1  x	27128 Erasable Programmable Read Only Memory 250nS  ST Microelectronics M27128AF1   RS part No. 659-501

IC5,6,7                 3  x	Z80A 6MHz Parallel In and Out VLSI 			QFP package is availabe from Zilog Z84C2010FEC.

see file in ZIP file called PIOSMT.JPG for pin out information

on the QFP package for this Chip.

IC10,25                	2  x	74LS138 3 line to 8 line decoder/driver		SMT SOIC package available from Texas Instruments

0.1" version is RS part no. 307-648

IC11,12,               2  x	NE555N Programmable Timer			Harris 7555CBA SMT version. RS part No. 184-0871

pin out is as 0.1" version.

IC13,14,15,16,      4  x	74HCT688 8 bit comparator with enable		Texas Instruments CD74HCT688MSMT.  RS No. 833-951

pin out for 0.1 format is available in hct688.JPG.

IC17,18,                2  x	74HCT245 octal 2 way transciever			Texas Instruments CD74HCT245MSMT .  RS Part No. 833-793

IC20,19,                1  x	74HCT273 octal D-type latch			Texas Instruments CD74HCT273MSMT .  RS Part No. 833-816

IC23,                     1  x	 HT6547B  Keyboard Encoder			available from Holtek Electronics, Taiwan. original sourced with Maplin.

Maplin no longer keep IC. contact www.holtek.com.tw and give your

order details. they are most helpfull. ask for shipping costs to UK.

ask them for UK distributor list if need be.

available as a 40 pin DIP. pin out data in holtek.JPG

IC4,24,                  2  x	CD4067BE 16 line to 1 line switch array		Texas Instruments CD4067ESMT . 0.1" version is RS No. 299-3724

IC22,                     1  x	CD4016BE quad PTMRTB switch array		Texas Instruments CD4016ESMT .0.1" version is RS No. 304-0540

IC8,                       1  x	CD4053BE tripple DPST switch array		Texas Instruments CD4053ESMT . 0.1" version is RS No. 304-0720

IC21,                     1  x	7447  7 Segment Display Decoder/Driver		Texas Instruments SN7447N RS No. 305-406.  use DIL package. IC

is mounted on its own PCB with some other compoonents.

to see 7 seg PCB artwork look at 7segPCB.JPG

to see connections and component overlay see segPCBco.JPG.

IC9,                       1  x	74LS04  Hex Inverter				Texas Instruments SN74LS04NSMT. 0.1" version is RS No. 307-503

TR9,10,11,12,7     5  x	2SC945/BC167B  NPN hi gain bipolar transistor	package is TO92. no RS code.

TR1,2,3,4,             4  x	BC109C						ST Microelectronics TO18c. RS Part No. 293-549.

TR5,                      1  x	AC179 or any PNP Silicon equiv			could use Toshiba 2SA1370  TO92a.  RS No 198-0008

TR6,                      1  x	BC107B						package is TO18c by ST Microelectronics. RS part No. 293-527

TR8,                      1  x	2N3702 or 3  PNP hi gain bipolar transistor		package is TO92a.

version 2N3703 is by various makers. RS No. 264-052.  TO92a

LD1,2,3,4,5,6,       6  x	xtra brite Light Emmiting diodes Various colour 	Siemens 2 x 1.4mm Hyper Bright TOPLED SMT. RS No's:

LD1 = 284-1106 Red, LD2 = 284-1112 Yellow, LD3 = 284-1083 Green

LD4 = 284-1106 Red, LD5 = 284-1112 Yellow, LD6 = 284-1083 Green

D1,2,                     2  x	Germanium Signal Diodes (any cat's whisker type)	use OA91 or equivalent. can  use SMT 1N4148 silicon.

1N4148  with leads - RS No. 271-606 various makes.

D3,                        1  x	1N4001 Silicon Power Diode			General Semiconductor SS16 . RS Part No. 269-833

X1,                        1  x	3.2768 Mhz Quartz Crystal			use appropriate SMT Crystal I have no information.

RESISTORS

R3,14,15,16,17,18,19,

20,21,22,23,24,25,26,

27,28,50,52,59,60,65,

29,	22x	  10k		RS No. 223-0562

R1,2,4,5,39,40,41,42,

57,56,53,58,61,62,63,

64,	16x	  1k		RS No. 223-0427

R55,44,45,46,47,      	5 x             220R		RS No. 223-0332		all are by Neohm..

R48,	1 x	  22R		RS No. 223-0203		type:    SMT 0805 1% 0.125w Chip resistors. Thich Film

R49,	1 x	  220k		RS No. 223-0742

R51,	1 x	  100k		RS No. 223-0691

R54,	1 x	  68k		RS No. 223-0663

R43,	1 x	  100R		RS No. 223-0297

R55,38,37,36,35,34,

33,32,10,	9 x	  470R		RS No. 223-0376

R31,	1 x	  330R		RS No. 223-0354

R30,	1 x	  8K2		RS No. 223-0556

R66,	1 x	  4k7		RS No. 223-0528

R13,	1 x	  2k7		RS No. 223-0483

R12,11,6,	3 x	  2k2		RS No. 223-0477

R9,	1 x	  820k		RS No. 223-0815

R8,	1 x	  2k  		RS No. 240-1691		Bourns 4mm Semi-Sealed 250mW Cermet SMT Trimmer

R7,	1 x	  47k		RS No. 223-0641

64 resistors, 1 variable

CAPACITORS

C1,2,3,5,6,7,8,9,15,14,

10,11,12,13,16,17,18,19,

C21,22,23,24,41,	23 x	0.1 uF 25v Miniature Disc Ceramic		Kemet Electronics 1206 SMT Capacitors Z5U dielectric

RS No. 264-4191

C27,	1 x	47 uF 6.3v Radial Electroylitic		National Panasonic Series HB Aluminium Electroylitic

RS No. 197-8320

C28,29,31,30,	4 x	47 pF Tubular or Disc Ceramic		Kemet Electronics 1206 SMT Capacitors COG dielectric

RS No. 298-9141

C32,	1 x	82 pF Tubular or Disc Ceramic		Murata 0705  SMT Capacitors COG dielectric

RS No.  237-6882

C26,	1 x	820pF Disc Ceramic			Murata 0705 SMT Capacitors COG dielectric

RS No. 237-6955

C25,4,43,20,	4 x	10 uF 35v Radial Electroylitic		National Panasonic Series HB Aluminium Electroylitic

RS No. 197-8409

C36,	1 x	0.33 uF 25v Radial Tantalum 		Siemens Series B45196 SMT Solid Tantalum Chip capacitor

RS No. 203-1955

C37,38,	2 x	1 uF 50v Radial Electroylitic		National Panasonic Series HB Aluminium Electroylitic

RS No. 197-8465

C39,	1 x	10 nF 25v Miniature Disc Ceramic		Kemet Electronics 1206 SMT Capacitors X7R dielectric

RS No. 298-9220

C35,	1 x	47 nF Disc Ceramic			Kemet Electronics 1206 SMT Capacitors X7R dielectric

RS No.  298-9242

C33,34,42,	3 x	100 uF 6.3v Radial Electroylitic		National Panasonic Series HB Aluminium Electroylitic

RS No. 197-8336

C40, 	1 x	2n2F Disc Ceramic			Kemet Electronics 1206 SMT Capacitors COG dielectric

RS No. 264-4090

for all electroylitics, see Electsmt.JPG for a close up of the structure of these type of capacitors.

INDUCTORS

L1,2,	2 x	100 uH Choke				EPCOS (formerly Siemens) SIMID 02 SMT Inductor

RS No. 191-0194

RLY1,	1x	12v Coil DPCO,G6H-210 1A Omron SMT Relay RS No. 196-6119

LS1,	1x	20mW Loudspeaker or Piezo Insert		Murata vertically mounted PCB type Transducer

Murata model No. PKM17EPT-4001

RS No. 189-5039

MISCELEANOUS

SW4			1 x	push to make release to break single pole switch		Apem Panel mount pushbutton normally open

RS No. 103-6296

RB1,			1 x	1k x 8 way Bussed Resistor block			Bourns Series 4816P Resistor Network pack

SMT mount. RS No. 136-525

PLP1,			2 x	need 1x 12 way for PIO 1 port B,

1 x 7 way for the 7 Seg board

and the system reset push switch SW4.		Molex through board connectors.

12 way Header = RS no. 53398-1290

12 way receptacle housing RS No. 51021-1200

7 way Header = RS No. 53398-0790

7 way receptacle housing RS No. 51021-0700

3 x	DS/SS Glass Fibre Copper Clad Photo Boards:

SS 7 Seg to dimensions shown in drawing.		see SegPCBco.JPG for component information

see 7SegPCB.JPG for PCB artwork information

simply print the Artwork out so that you can

accurately line up the pins of a 7447 chip on the

printout, you now have the LWH dimensions of the

board.

SS power control board to dimensions in drawing.	see POWSW.pcx for PCB artwork information

see powswcom.JPG for component information

aquire LWH dimensions in the same manner as above.

DS SMT main board to your discretion.			approximate size:

L = 160mm x W 90mm if using SMT.

1 x	4 meters of mono coax 1.5mm diameter			RS No. 290-9145   305 metres, White coax

by Belden

1 x	5 way DIN plug & lead to 6 way Mini DIN plug		5 way DIN plug 180 deg RS No. 234-4188 various makes.

4 way Multicore coax & data sheilded cable.

7 way (only 4 way needed) 100m RS No. 368-362

6 way Mini DIN plug. RS No. 463-388 various makes

1 x	4 way DIN socket & lead with 6 way Mini DIN plug		RS No. 253-709 by Cherry.

or Keyboard Adaptor Mark I

SWB1,2,			2 x	DIL switch blocks 8 way				RS No. 183-846 8 way various makes.

JKSK1,2,3,			3 x	3.5mm Mono Jack sockets PCB mount			dont have any information on this.

3 off,			3 x	3.5mm Mono Jack plugs	 			RS No. 449-994 insulated plug vaious makes

MDS1,2,			2 x	6 way Mini DIN PCB Mount Socket			RS No. 463-467 various makes

SW1,(power),		1 x	5A illuminated, DPDT Mains Switch Panel Mount		RS No. 250-7929 latching type, Red. various makes

MIC1,			1 x	600 Ohm Moving Coil Dynamic Microphone Insert		Maplin QY62S electret condenser insert.

if condenser Mic used, C38 should be omitted.

7SEG,			1 x	7 Segment LED Display Common Anode, 25mm type  	use a 25mm Toshiba TLG306 or equiv

7SegPCB.JPG for Artwork of its board and

SegPCBco.JPG for connection details.

1 x	Hard Plastic Box					to your discretion. to fit the PSU and the power

control boards together. example:   RS No. 243-3711

RFi screened Box IP65 by Rolec

FS1,			1 x	20mm Fuse Mount PCB type				to fit 500mA F type 25mm Glass Fuse. RS No.

Schurter dual size OGD PCB mount fuse holder

RS No. 336-7851

SW2(power)			1 x	SPDT rocker switch with centre off.			RS No. 199-122 various makes

1 x	BS1363/A Cold socket, lead, and 13A Plug		RS No. 426-418 various makes

1 x	BS1363/A cold socket, lead, and cold plug		RS No. 487-277  IEC/CE 22 moulded plug & socket

PAN1,(power)		1 x	BS1363/A cold inlet/outlet panel, fused			RS No. 211-1017 fused. various makes

SMPS1,(power)		1 x	SMPS PSU LPT-22 25 Watts, PCB type, 12v 1.5A		by Astec Standard Power. RS No. 192-1087

and 5v 3A					Standard PCB type ready made SMPS unit.

Cable Grommets Panel Mount 3 mm Diameter		example: RS No. 543-197

1 x            Pack of 100 Nylon Cable Ties.			example: RS No. 255-2806

small cable ties for tying signal wires in PSU/Control Box.

HARDWARE FOR THE MAINS CIRCUITRY

R1,	1 x	6K8  8 Watt Power Resistor			RS No. 252-2704, made by Arcol

HTK1,	1 x	5.5 degrees C/Watt Resistor Heatsink		RS No. 234-2378 various makes

R2,	1 x	51R  0.25 Watt Resistor metal film			RS No. 148-180 various makes

R3,	1 x	68R  0.25 Watt Resistor metal film			RS No. 148-219

R4,	1 x	1K 0.25 Watt Resistor metal film			RS No. 148-506

D2,	1 x	1N4001 Silicon Power Diode			RS No. 261-148 by General Semiconductor

D1,	1 x	BZX852V7 1.3W Zener Diode			RS No. 812-320 by Motorola

C1,	1 x	2200uF 16v Electroylitic				RS No. 493-711 105 degC Axial by Hua Lien

OPT1,	1 x	OPI110C Opto Isolator, Axial mount			RS No. 194-571 by Bedford Opto Technology

see file OPI110.JPG for mechanical information

THY1,	1 x	 BTA12-600BW snubberless, isolated TRIAC		RS No. 659-961by ST Microelectronics

see file Triac.JPG for electrical information

uses standard in line 3 leg 0.1 spacing.

C2,3,	2 x	0.22uF Type X2 275VAC Suppression 		RS No. 311-1052 by Evox-Rifa PME 271MD Series

standard 0.1 footprint.

L1,	1 x	High Current PCB Mount Choke			Roxburgh Electronics Ltd, Open former, differential

mode SMV Series. RS No. 188-9078  1 Ampere.

>>>>>THIS CONCLUDES THE HARDWARE LIST FOR THE Z80 FOR PC DESCATOC CARD<<<<<